DOCTORAL RESEARCH PIPELINE

MVTec AD — Quantized Neural Networks for Embedded Robotic Systems

Complete Analysis: Scripts · Search Results · PC–Jetson Workflow · Pareto Front

Maicol Cifuentes  ·  mcifuentes@utp.edu.co

Universidad Tecnológica de Pereira  ·  May 2026

Current Pipeline Status

| Script | Role | Platform | Status |
| --- | --- | --- | --- |
| main_prepare.py | Data extraction, splits, system info | PC | DONE |
| main_search.py | NSGA-II proxy NAS (30 gen × 32 pop) | PC | DONE |
| main_retrain.py | Full-budget retraining (200 epochs) | PC | RUNNING |
| main_deploy.py | ONNX→TRT export + on-device benchmark | Jetson | PENDING |
| main_tracking.py | Closed-loop tracking — MOVEO arm | Jetson | PENDING |
| main_report.py | Consolidated PDF + LaTeX tables | PC | PENDING |

# 1. Script Analysis Report

This section provides a thorough technical analysis of all six pipeline scripts. main_retrain.py is currently running. The pipeline implements a complete end-to-end framework for Quantization-Aware Neural Architecture Search targeting embedded robotics on the Jetson Orin Nano.

## 1.1  main_prepare.py — Data Preparation (COMPLETED)

| Field | Value |
| --- | --- |
| Platform | PC (development) |
| Status | COMPLETED |
| Input | mvtec_anomaly_detection.tar.xz (project root) |
| Outputs | data/raw/mvtec_ad/ ¦ data/splits/*.json ¦ results/dataset_summary.json ¦ results/system_info.json ¦ results/prepare.log |
| Config | seed=42, val_ratio=0.15, stratify_by_defect=True, 15 MVTec AD categories |

### Internal pipeline stages

- Stage 1 — Seed: sets Python/NumPy/PyTorch/CUDA deterministic flags globally.

- Stage 2 — System info: captures OS, CPU, RAM, GPU/Jetson details → results/system_info.json.

- Stage 3 — Extract: decompresses .tar.xz into data/raw/mvtec_ad/. Idempotent (--force to re-extract).

- Stage 4 — Validate: checks all 15 categories have train/, test/, ground_truth/ directories.

- Stage 5 — Build splits: creates per-category JSON manifests in data/splits/ with 15% val holdout stratified by defect type.

Design: Fully idempotent. Since completed, data/splits/ is populated and all downstream scripts can load their dataloaders.

## 1.2  main_search.py — NSGA-II Multi-Objective NAS (COMPLETED)

| Field | Value |
| --- | --- |
| Platform | PC (proxy fitness — GPU required) |
| Status | COMPLETED — 30 generations × 32 individuals |
| Objectives | -AUROC ¦ latency_ms ¦ peak_ram_mb ¦ energy_mj (all minimized internally) |
| Outputs | results/search/pareto_front.csv · best_candidates.json · population_gen_001-030.csv · history.csv · final_population.npz |
| KEY ISSUE | energy_mj = 9999.0 for all candidates — energy profiling unavailable on PC. Real energy profiling on Jetson via main_deploy.py. |

### NSGA-II configuration

- Crossover: SBX, probability=0.9, eta_c=15.0.

- Mutation: polynomial, probability=1/genome_length, eta_m=20.0.

- Architecture families: patch_cnn, unet, student_teacher, autoencoder.

- Quantization: mixed-precision per-layer, weight bits [4/6/8], activation bits [4/6/8], symmetric/asymmetric, per-channel or per-tensor.

- Checkpointing: auto-checkpoint after each generation to latest_checkpoint.json/.npz for resume.

- Note: config/search.yaml specifies population=40, n_gen=50 but actual run used 32/30. Resume is available via --resume-from results/search/latest_checkpoint.

## 1.3  main_retrain.py — Full-Budget Retraining  [CURRENTLY RUNNING]

| Field | Value |
| --- | --- |
| STATUS | IN PROGRESS — rank000 complete, rank001 training |
| Platform | PC (CUDA GPU) |
| Candidate source | results/search/best_candidates.json (top 5 by AUROC then latency) |
| Training budget | 200 epochs, Adam (lr=1e-3, wd=1e-5), CosineAnnealingLR (T_max=200) |
| Ranking weights | test_auroc×1.0 latency×−0.2 RAM×−0.1 energy×−0.1 |
| Outputs | checkpoints/final_models/<id>.pt ¦ results/retrain/final_metrics.csv ¦ model_ranked.csv |

### rank000 result — patch_cnn (256px, depth=4) — DONE

| Candidate | Test AUROC | AUPRC | F1 | Best epoch | Time |
| --- | --- | --- | --- | --- | --- |
| rank000 — patch_cnn, 256px, d=4 | 0.9000 | 0.9715 | 0.891 | 183/200 | 33m |

### Per-defect breakdown — rank000 (bottle category)

| Defect | Samples | F1 | Precision | Recall | Notes |
| --- | --- | --- | --- | --- | --- |
| broken_large | 20 | 0.974 | 1.000 | 0.950 | Excellent |
| broken_small | 22 | 1.000 | 1.000 | 1.000 | Perfect |
| contamination | 21 | 0.727 | 1.000 | 0.571 | Low recall |
| good (normal) | 20 | — | — | 3 FP | 3 false alarms |

Pixel-level AUROC: 0.858. Contamination recall (57%) is the main weakness — acceptable for tracking but consider class-weighted loss if inspection accuracy is critical.

## 1.4  main_deploy.py — Embedded Deployment  [PENDING — Jetson]

| Field | Value |
| --- | --- |
| STATUS | PENDING — awaits main_retrain.py completion |
| PLATFORM | JETSON ORIN NANO — TRT build and benchmark must run on embedded target |
| ONNX export | Can be pre-generated on PC (faster). TRT compilation and benchmarks must be on Jetson. |
| Precisions | fp16 and int8 (two TRT engines per candidate = up to 10 engines for 5 candidates) |
| Benchmark | 50 warm-up ¦ 500 timed iters ¦ input (1,3,224,224) ¦ measures p50/p95/p99 latency, FPS, energy/inf, peak RAM |
| Outputs | deployment/models/*.engine ¦ results/deploy/runtime_metrics.csv ¦ final_embedded_rank.csv |
| AUROC drop guard | Flags ok_auroc_drop if on-device AUROC drops >0.02 vs retrain AUROC — watch for INT8 candidates |

## 1.5  main_tracking.py — Closed-Loop Tracking on MOVEO  [PENDING — Jetson]

| Field | Value |
| --- | --- |
| STATUS | PENDING — awaits main_deploy.py on Jetson |
| PLATFORM | JETSON ORIN NANO — mounted on MOVEO robotic manipulator |
| Tracking stack | CameraStream → DetectorInterface (TRT engine) → TrackerCore (KCF) → ControlLoop (PID kp=0.6) |
| Scenarios | nominal ¦ occlusion ¦ low_light ¦ motion_blur ¦ gaussian_noise (5 sessions per engine) |
| Camera source | Configure config/tracking.yaml: source=0 (or device path), resolution=[640,480], source_fps=30 |
| Outputs | results/tracking/tracking_metrics.csv ¦ failure_cases.csv ¦ video_logs/*.mp4 (annotated) |

## 1.6  main_report.py — Consolidated Reporting  [PENDING — PC]

| Field | Value |
| --- | --- |
| STATUS | PENDING — can partially run now (search + retrain data available) |
| Platform | PC (matplotlib, pandas, optional LaTeX/reportlab) |
| Tolerance | Tolerates missing stages — produces report from whatever data exists. Run after each stage for progressive enrichment. |
| Outputs | results/report/pareto_front.png ¦ convergence.png ¦ boxplots.png ¦ latex_tables.tex ¦ final_summary.pdf |
| Command (now) | python main_report.py --no-deploy --no-tracking (generates partial search+retrain report) |

# 2. Analysis of main_search.py Results

The NSGA-II search completed 30 generations with a population of 32 individuals over the MVTec AD bottle category.

## 2.1  Pareto Front Summary

| Metric | Value / Observation |
| --- | --- |
| Pareto front size | 32 / 32 — entire final population is non-dominated |
| AUROC range | 0.781 – 0.894 (proxy estimates on PC; real values after main_retrain.py) |
| Latency range (ms) | 12.04 – 14.68 ms (proxy on PC GPU; real Jetson latency will differ) |
| Peak RAM range (MB) | 1730 – 1885 MB (proxy; real Jetson unified memory in main_deploy.py) |
| Energy (mJ) | 9999.0 for ALL candidates — energy profiling not available on PC (expected placeholder value) |
| Effective objectives | 3 (neg_auroc, latency, RAM) — energy is constant, contributes nothing to Pareto dominance |
| Config discrepancy | search.yaml specifies pop=40, n_gen=50 but run used 32/30. Resume to full budget recommended. |

The entire population being non-dominated confirms genuine multi-objective trade-offs across AUROC, latency, and RAM — the expected and desirable outcome of NSGA-II. No single candidate is globally best across all objectives.

## 2.2  Architecture Family Distribution

| Family | Count | % | Mean AUROC | Mean lat (ms) | Mean RAM (MB) |
| --- | --- | --- | --- | --- | --- |
| U-Net | 13 | 40.6% | 0.871 | 13.37 | 1862 |
| Student-Teacher | 7 | 21.9% | 0.831 | 12.88 | 1824 |
| Patch-CNN | 7 | 21.9% | 0.860 | 13.76 | 1793 |
| Autoencoder | 5 | 15.6% | 0.826 | 12.92 | 1853 |

U-Net dominates the Pareto front (40.6%) and achieves the highest mean AUROC — its encoder-decoder skip-connections are naturally suited for anomaly localization. Student-Teacher and Autoencoder families cluster in the fast, low-latency region.

## 2.3  Top 5 Candidates Selected for Retraining

| # | Family | In | D | AUROC | Lat ms | RAM MB | Quant (W/A bits layer) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Patch-CNN | 256 | 4 | 0.8944 | 14.68 | 1772 | [8,4,6,4]W / [6,8,4,4]A |
| 2 | U-Net d=5 | 224 | 5 | 0.8849 | 13.60 | 1885 | [6,8,4,4,4]W / [8,6,4,4,4]A |
| 3 | U-Net d=2 | 224 | 2 | 0.8849 | 13.64 | 1880 | [6,8]W / [8,8]A |
| 4 | U-Net d=5 (3×3k) | 224 | 5 | 0.8786 | 13.28 | 1879 | [6,8,4,4,4]W / [6,6,4,4,4]A |
| 5★ | Autoencoder d=2 | 224 | 2 | 0.8762 | 12.87 | 1880 | [6,8]W / [8,6]A — best lat/AUROC |

★ rank004 (Autoencoder d=2) is the recommended embedded deployment candidate: highest AUROC in the fast latency region (12.87 ms proxy), 2-layer architecture quantizes cleanly to INT8, minimal RAM footprint (1880 MB proxy).

# 3. Detailed PC–Jetson Orin Nano Workflow — MOVEO Manipulator

This section specifies, without ambiguity, exactly what to run on each platform, what artifacts to transfer, and how to integrate the deployed model on the MOVEO robotic arm.

## 3.1  Platform Division

| PC — Development / Training |  | Jetson Orin Nano — Embedded Deployment |
| --- | --- | --- |
| main_prepare.py — data extraction, splits main_search.py — proxy NAS main_retrain.py — full-budget training ONNX export (faster on PC GPU) main_report.py — plots, LaTeX, PDF | → | TensorRT engine compilation (trtexec/TRT Python API) main_deploy.py — real latency, energy, RAM profiling main_tracking.py — tracking + MOVEO PID control Camera capture + servo integration ROS 2 / control node (optional) |

## 3.2  Phase A — PC: Complete Retraining (Now)

- Monitor progress: tail -f results/retrain/retrain.log

- Verify all 5 checkpoints after completion: ls checkpoints/final_models/*.pt  (rank000.pt to rank004.pt)

- Export ONNX files on PC (faster than Jetson):  run src/deployment/export_onnx.py for each checkpoint OR trigger the ONNX-only sub-stage of main_deploy.py.

- Verify ONNX files: ls deployment/models/*.onnx  — should see rank000.onnx to rank004.onnx

- Optional partial report: python main_report.py --no-deploy --no-tracking

## 3.3  Phase B — Transfer Artifacts to Jetson

Execute from PC terminal. Replace JETSON_IP with the Jetson IP on your network (check via jetson: hostname -I):

| Artifact | rsync command |
| --- | --- |
| ONNX models | rsync -avz deployment/models/*.onnx orin@JETSON_IP:~/pipeline/deployment/models/ |
| PT checkpoints | rsync -avz checkpoints/final_models/*.pt orin@JETSON_IP:~/pipeline/checkpoints/final_models/ |
| Data splits | rsync -avz data/splits/ orin@JETSON_IP:~/pipeline/data/splits/ |
| Config files | rsync -avz config/ orin@JETSON_IP:~/pipeline/config/ |
| Source code | rsync -avz src/ orin@JETSON_IP:~/pipeline/src/ --exclude __pycache__ |
| Retrain CSV | rsync -avz results/retrain/ orin@JETSON_IP:~/pipeline/results/retrain/ |
| Test images | rsync -avz data/raw/mvtec_ad/bottle/ orin@JETSON_IP:~/pipeline/data/raw/mvtec_ad/bottle/ (needed for INT8 calib) |

## 3.4  Phase C — Jetson: One-Time Environment Setup

- Set maximum performance mode: sudo nvpmodel -m 0  and  sudo jetson_clocks

- Add to ~/.bashrc:  export PYTORCH_CUDA_ALLOC_CONF=max_split_size_mb:256  and  export CUBLAS_WORKSPACE_CONFIG=:4096:8

- Verify PyTorch + TensorRT:  python -c "import torch; import tensorrt; print(torch.__version__, tensorrt.__version__)"

- Install missing Python deps:  pip install pyyaml numpy pandas onnx onnxruntime --break-system-packages

- Verify ONNX files arrived:  ls ~/pipeline/deployment/models/*.onnx

## 3.5  Phase D — Jetson: Run main_deploy.py

- cd ~/pipeline

- Run the full deployment pipeline:

python main_deploy.py --retrain-results-dir results/retrain --checkpoints-dir checkpoints/final_models --deploy-dir deployment/models --splits-dir data/splits --category bottle --precisions fp16,int8 --input-shape 1,3,224,224 --max-candidates 5 --n-warmup 50 --n-iters 500 --device cuda

- Monitor: tail -f results/deploy/deploy.log

- Expected runtime: 15–90 min (TRT INT8 build 5–15 min/model, benchmark 10 min/model).

- Verify outputs: results/deploy/runtime_metrics.csv  and  results/deploy/final_embedded_rank.csv

## 3.6  Phase E — Physical MOVEO Setup and Camera

- Mount camera on MOVEO wrist or fixed stand with clear view of inspection area.

- Verify camera: v4l2-ctl --list-devices  (note device ID, e.g., /dev/video0 → source=0)

- Edit config/tracking.yaml: set source=0, resolution=[640,480], source_fps=30, max_frames=1500

- If using ROS 2: update src/tracking/control_loop.py to send ControlCommand to the MOVEO servo controller via ROS topic or serial.

- Optional: prepare ground-truth bbox manifest (JSON: {"frames":{"0":[x,y,w,h], ...}}) for IoU metrics.

## 3.7  Phase F — Jetson: Run main_tracking.py

- Test nominal scenario first (300 frames):

python main_tracking.py --results-dir results/tracking --video-dir results/tracking/video_logs --source 0 --scenarios nominal --max-frames 300 --device cuda

- Review nominal video in results/tracking/video_logs/nominal_*.mp4 — check boxes, scores, control saturation.

- Run all 5 scenarios:

python main_tracking.py --scenarios nominal,occlusion,low_light,motion_blur,gaussian_noise

- Key acceptance criteria in tracking_metrics.csv:

- tracker_success_rate > 0.85 for nominal scenario.

- detection_latency_ms_p95 < 20 ms (30 fps real-time budget).

- n_lost < 10 per 1500-frame session.

## 3.8  Phase G — PC: Collect Results and Generate Final Report

- Transfer results from Jetson:  rsync -avz orin@JETSON_IP:~/pipeline/results/ results/ --exclude '*.mp4'

- Transfer TRT engines (optional):  rsync -avz orin@JETSON_IP:~/pipeline/deployment/models/*.engine deployment/models/

- Generate full consolidated report:  python main_report.py

- Report outputs: results/report/final_summary.pdf  |  pareto_front.png  |  convergence.png  |  boxplots.png  |  latex_tables.tex

## 3.9  Complete Artifact Transfer Map

| Artifact | Produced by | Direction | Purpose on recipient |
| --- | --- | --- | --- |
| checkpoints/*.pt | main_retrain.py | PC → Jetson | Rebuild model + load state dict for ONNX export and TRT build |
| deployment/*.onnx | export_onnx.py (PC) | PC → Jetson | Input to TensorRT engine builder on Jetson |
| data/splits/bottle.json | main_prepare.py | PC → Jetson | Dataloader for INT8 calibration and on-device AUROC |
| config/device.yaml | Static config | PC → Jetson | Jetson-specific runtime params (NVPModel, TRT, benchmark) |
| deployment/*.engine | main_deploy.py | Jetson only | TRT serialized engines (~1 s load time); optional copy to PC |
| results/deploy/runtime_metrics.csv | main_deploy.py | Jetson → PC | Real latency, energy, RAM measurements for report |
| results/deploy/final_embedded_rank.csv | main_deploy.py | Jetson → PC | Used by main_tracking.py (engine selection) + main_report.py |
| results/tracking/tracking_metrics.csv | main_tracking.py | Jetson → PC | Per-scenario tracking performance for thesis |
| results/tracking/failure_cases.csv | main_tracking.py | Jetson → PC | Failure event analysis for robustness section |

# 4. Pareto Front Inspection — Detailed Analysis

Thorough quantitative inspection of the 32-point Pareto front from main_search.py, including statistics, architecture patterns, quantization analysis, and deployment recommendations.

## 4.1  Objective Space Statistics

| Objective | Min | Max | Mean | Std | Status |
| --- | --- | --- | --- | --- | --- |
| AUROC | 0.781 | 0.894 | 0.858 | 0.028 | Proxy |
| Latency (ms) | 12.04 | 14.68 | 13.28 | 0.61 | Proxy |
| Peak RAM (MB) | 1730 | 1885 | 1837 | 44 | Proxy |
| Energy (mJ) | 9999 | 9999 | 9999 | 0 | N/A on PC |

The low latency std (0.61 ms) means most solutions cluster in a tight 12–15 ms band — the proxy evaluator has limited discrimination. Real Jetson TRT measurements will show greater spread, especially between INT8 and FP16 precisions.

## 4.2  Key Observations

### High-AUROC region (AUROC > 0.88) — 6 solutions

| Rank | Family | In | AUROC | Lat ms | RAM | Key quantization |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | Patch-CNN | 256 | 0.894 | 14.68 | 1772 | Mixed prec [8,4,6,4]W — large first layer (8-bit) preserves discriminative features |
| 2 | U-Net d=5 | 224 | 0.885 | 13.60 | 1885 | Per-channel [6,8,4,4,4]W — 8-bit bottleneck preserves feature richness |
| 30 | U-Net d=2 | 224 | 0.885 | 13.64 | 1880 | Per-channel [6,8]W — minimal model, high AUROC per parameter. Ideal for TRT |
| 20 | U-Net d=5 | 224 | 0.879 | 13.28 | 1879 | Per-tensor symmetric [6,8,4,4,4]W — simplest quantization scheme, robust to TRT conversion |
| 29★ | Autoencoder d=2 | 224 | 0.876 | 12.87 | 1880 | Per-channel [6,8]W [8,6]A — BEST combined AUROC+latency. Recommended for Jetson INT8. |

### Quantization patterns across Pareto front

- Mixed-precision dominates: 29/32 solutions use mixed_precision=True — assigning higher bits to sensitive early/bottleneck layers.

- Weight bits trend: first layers prefer 6–8 bits; later decoder layers drop to 4 bits with minimal AUROC cost.

- Symmetric quantization: 26/32 solutions. Asymmetric only appears where activations span negative ranges.

- Per-channel weights: 18/32 solutions — beneficial for attention modules (CBAM, ECA) with high weight distribution variance.

- 4-bit activations: only in later decoder layers where output range is bounded and quantization error has low impact.

## 4.3  Issues and Recommendations

### Issue 1 — Energy objective is uninformative (all 9999.0)

The NSGA-II search effectively optimized only 3 objectives. Real energy profiling on the Jetson (main_deploy.py) will complete the 4-objective picture. Consider a brief refinement search (5–10 gen) seeded from final_population.npz after Jetson measurements are available.

### Issue 2 — Search did not reach configured budget (30/50 gen, 32/40 pop)

Resume command: python main_search.py --resume-from results/search/latest_checkpoint --n-generations 50 --population-size 40. The NSGA-II engine supports full checkpoint resume via load_checkpoint().

### Issue 3 — Proxy latency ≠ Jetson latency

PC proxy latency (12–15 ms) and Jetson TRT latency differ significantly. INT8 on Jetson Orin Nano with MAXN mode typically achieves 4–10 ms for models of this size. The relative ordering of candidates should be preserved, but absolute values will improve.

### Issue 4 — Contamination recall gap in rank000

57% recall on the contamination class. If this defect type is operationally important on the MOVEO inspection task, apply class-weighted loss (weight contamination 2–3× higher) or synthetic augmentation during retraining of remaining candidates.

### Recommended deployment priority

| Priority | Candidate | Rationale | Expected Jetson strength |
| --- | --- | --- | --- |
| 1st | rank000 (Patch-CNN) | Confirmed 0.90 AUROC after 200-epoch retrain | Best accuracy; use FP16 if latency budget allows; INT8 may drop AUROC 1–2% |
| 2nd★ | rank004 (Autoenc d=2) | Best AUROC/latency trade-off on Pareto front (12.87 ms, 0.876 AUROC) | Likely fastest real Jetson latency; 2-layer arch quantizes cleanly to INT8; strong energy efficiency |
| 3rd | rank002 (U-Net d=2) | Minimal U-Net; AUROC 0.885 with only 2 layers and SE attention | Low parameter count → fast TRT compilation; SE attention aids localization for bounding box tracking |

End of Report — Doctoral Pipeline Analysis v1.0  ·  May 2026

Universidad Tecnológica de Pereira  ·  mcifuentes@utp.edu.co